| Princeton Plasma Physics Laboratory NSTX-U Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Relationship Between Plasma Turbulence and SOL Width Scaling in NSTX-U |  |  |  |
| OP-XP-1514 | Revision: 1 | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: TK Gray |  |  | Date |
| SG, TSG or TF Leader (assigned by RC): V Soukhanovskii |  |  | Date |
| Run Coordinator (RC): J Menard |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| RESTRICTIONS or MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX- EXPERIMENTAL PROPOSAL

| TITLE: Relationship Between Plasma Turbulence and SOL Width Scaling in NSTX-U | No. OP-XP-1514 |
| --- | --- |
| AUTHORS: TK Gray, S Zweben | DATE: |

# 1.	Overview of planned experiment

The goal in this experiment is to measure the variations in the SOL width observed under various NSTX-U parameters and correlate that with measurements of SOL turbulence at the midplane and in the divertor as a function of PNBI, Ip/Bt and varying amounts of pre-discharge Li deposition.

# 2.	Theoretical/ empirical justification

The use of lithium wall conditioning in NSTX resulted in a reduction of SOL width, λq as measured by divertor infrared (IR) cameras.  Modeling performed by the Lodestar group suggests this contraction is due to reduced interchange turbulence, which is brought about because of the reduced pedestal density gradient that occurs with the addition of lithium.  Modeling with the XGC1 code predicts strong neoclassical effects are responsible for the contraction of λq. This experiment seeks to measure midplane SOL turbulence as measured by Gas Puff Imaging (GPI), Beam Emission Spectroscopy (BES) and Reflectometry and compare those data with Langmuir probe, visible imaging (tangential and perpendicular views), and IR thermography measurements in the lower divertor at several combinations of Ip/Bt, injected beam powers and Li depositions.

# 3.	Experimental run plan

This proposal is separated into two parts.  Part 1 uses minimal Li wall conditioning with evaporation rates between 10 – 50 mg of Li depending on what’s required for the discharge to run successfully.    The second part of the proposal repeats the first part but substantially increases the amount of pre-discharge Li evaporation to ~ 300 mg.  The actual amount of Li evaporation used will depend on the machine conditions on the day of experiment.  Table 1 summarizes the proposed shot plan.

It’s preferred to utilize all available NBI #1 sources prior to using sources from NBI #2.  When sources from the second NBI line are used, short (~20ms) pulses of source 1B are synchronized every ~100ms with notches of the second NBI sources to obtain time evolution of ion temperature, toroidal rotation and Zeff from CHERS measurements. The optimum setup for NB blips/notches will be determined after the “CHERS assessment” XMP.

Part 1 – Minimal Li Evaporation (8 shots total)

- Establish H-mode fiducial discharge shape with the following conditions (1 shot):

  - Ip = 0.9 MA, BT = 0.45 T, PNBI = 4 MW and outer strike point placed on the row 2 tile of the outboard divertor (r ~ 0.65 m)

- Repeat this step 1 for reproducibility (1 shot).

- Increase beam power to 8 MW with low Li evaporation and take 2 shots at these conditions (2 shots)

  - Ip = 0.9 MA, BT = 0.45 T, PNBI = 8 MW

- Increase Ip and Bt to 1.3 MA and 0.65 T with PNBI = 4 MW (2 shots)

  - If machine conditions and administrative limits allow, operate at Ip = 1.5 MA and BT = 0.75 T

- Keep Ip = 1.3 MA(1.5 MA) and Bt = 0.65 T (0.75 T), increase beam power to 8 MW with low Li evaporation and take 2 shots at these conditions (2 shots)

Part 2 – High Li Evaporation (8 shots total)

- Repeats steps 1 – 5 with a large amount (~ 300 mg) of Li evaporation

  - See Table 1 for prioritization of shots in part 2

  - Allow 2 – 4 shots to adjust centerstack fueling when the Li evaporation is first increased.  Look to the fueling that was required for discharges in XP 1529 with similar Li deposition amounts as a guide.

Table 1 describes the matrix of desired shots, listed in priority order, for this proposal with 4 shots left as contingency to adjust center stack fueling and to achieve the highest power discharges when greater amounts of Li evaporation is begun.

Table 1: Proposed Shot Matrix listed in priority order and diagnostic check-list.  Diagnostics not listed below, that are listed as ‘Needed’ Diagnostic Checklist, are to be run every shot.  Contingency scenarios for the diagnostics listed below are given in Section 4.

| Minimal Li |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shot | Ip [MA] | Bt [T] | PNBI (MW) | GPI | SOL Refl. | Probes |
| 1 | 0.9 | 0.45 | 4 | Early | Swept | Swept |
| 2 | 0.9 | 0.45 | 4 | Late | Fixed Freq. | isat |
| 3 | 0.9 | 0.45 | 8 | Early | Swept | Swept |
| 4 | 0.9 | 0.45 | 8 | Late | Fixed Freq. | isat |
| 5 | 1.3/1.5 | 0.65/0.75 | 4 | Early | Swept | Swept |
| 6 | 1.3/1.5 | 0.65/0.75 | 4 | Late | Fixed Freq. | isat |
| 7 | 1.3/1.5 | 0.65/0.75 | 8 | Early | Swept | Swept |
| 8 | 1.3/1.5 | 0.65/0.75 | 8 | Late | Fixed Freq. | isat |
| Heavy Li |  |  |  |  |  |  |
| 9 | 0.9 | 0.45 | 4 | Early | Swept | Swept |
| 10 | 0.9 | 0.45 | 4 | Late | Fixed Freq. | isat |
| 11 | 1.3/1.5 | 0.65/0.75 | 4 | Early | Swept | Swept |
| 12 | 1.3/1.5 | 0.65/0.75 | 4 | Late | Fixed Freq. | isat |
| 13 | 0.9 | 0.45 | 8 | Early | Swept | Swept |
| 14 | 0.9 | 0.45 | 8 | Late | Fixed Freq. | isat |
| 15 | 1.3/1.5 | 0.65/0.75 | 8 | Early | Swept | Swept |
| 16 | 1.3/1.5 | 0.65/0.75 | 8 | Late | Fixed Freq. | isat |

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Needed XMPs: CHERS Assessment, Commission ISOFLUX and rtEFIT

Needed XPs: “Controlled introduction of Lithium into NSTX-U” (1529)

Desired, but not necessary XPs: “Investigating small-scale edge turbulence with GPI”

GPI requires a ratio of Ip (MA) / BT (T) = 2 for optimal viewing.  Table 2 is a list of viable Ip and BT combinations in NSTX-U to optimize the GPI view.  The XP titled “Investigating small-scale edge turbulence with GPI” will seek to optimize view in NSTX-U.  During operation of NSTX, GPI was typically operated with Ip = 0.9 MA and BT = 0.45 T.  While this is still an option in NSTX-U, machine and administrative conditions allowing, Ip and Bt may be increased up to 1.5 MA and 0.75 T respectively.

Diagnostics

| Given improvements in GPI collection optics for NSTX-U, it may be possible to run GPI with much less gas than was used in NSTX. Ip (MA) | BT (T) |
| --- | --- |
| 0.9 | 0.45 |
| 1.1 | 0.55 |
| 1.3 | 0.65 |
| 1.5 | 0.75 |

Table 2: Viable Ip and BT for optimal GPI view.

  - If this is the case, and the reduced gas puff from GPI doesn’t interfere with CHERS measurements, GPI should be run every shot rather than altering as listed in Table 1.

  - If this is not the case, GPI should be alternated between times ‘early’ and ‘late’ in the shot.  Delaying GPI until later in the shot will allow CHERS and BES measurements to get measurements ‘early’ in the shot.

    - Early ~ 500 ms

    - Late ~ 800 ms

    - Each GPI gas puff lasts for ~ 200 ms

  - The exact timing of the GPI puffs will be determined at the time of the XP

- SOL reflectometer will run either in fixed or swept frequency on alternating shots.

- Divertor Langmuir probes will alternate between fixed bias voltage (isat) for turbulence measurements or swept bias for divertor ne and Te

  - It may be possible to run ½ the probes in fixed bias and ½ with swept bias depending on availability of a 2nd bias power supply for the probes.  If this is the case, run the probes in this manner instead.

# 5.	Planned analysis

EFIT, SOLT, XGC1 and SOLPS

# 6.	Planned publication of results

Results allowing, this will be published at the next PSI and IAEA meetings.

# 7.	Estimated Neutron Production

Based on the number of shots, plasma current levels, and expected durations, estimate the maximum neutron production of this experiment. See calculator in Appendix #2 for this calculation.

# of Shots used in Estimate: 20			 Estimated Total Neutron Production: 2.2E16

PHYSICS OPERATIONS REQUEST

| TITLE: Relationship Between Plasma Turbulence and SOL Width Scaling in NSTX-U | No. OP-XP-1514 |
| --- | --- |
| AUTHORS: TK Gray, S Zweben | DATE: |

Brief description of the most important operational plasma conditions required and any special hardware requirement: The exact elongation and triangularity of the reference shot are not critical. More important, the shot should be an H-mode shot capable of running at [Ip,Bt] combinations as high as [1.3,0.65], or potentially [1.5,0.75], with up to 8 MW of power. The OSP shall be kept at 65 cm

Previous shot(s) which can be repeated: Previous shot(s) which can be modified:

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) BT Range (T): 0.45 – 0.65/0.75 Flattop Duration (s): 1 sec or greater IP Range (MA): 0.9 – 1.3/1.5 Flattop Duration (s): 1 sec or greater Configuration: LSN, drsep <= -10 mm Equilibrium Control: Isoflux (rtEFIT) with Strike-point control (rtEFIT) Outer gap (m): 10 cm Inner gap (m): Z position (m): Elongation: see above Triangularity (U/L): see above OSP radius (m): ~ 0.65 m Gas Species: D Injector(s): NBI Species: D Heating Duration (s): Voltage (kV) 50 cm (1C): 90 60 cm (1B): 90 70 cm (1A): 90 Voltage (kV) 110 cm (2C): 90 120 cm (2B): 90 130 cm (2A): 90 ICRF Power (MW): 0 Phase between straps (°): Duration (s): CHI: Off Bank capacitance (mF): LITERs: On Total deposition rate (mg/min) or dose per discharge (mg): Varies. See run plan for Li deposition amounts for each discharge. EFC coils: On

DIAGNOSTIC CHECKLIST [1]

| TITLE: Relationship Between Plasma Turbulence and SOL Width Scaling in NSTX-U | No. OP-XP-1514 |
| --- | --- |
| AUTHORS: TK Gray, S Zweben | DATE: |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Beam Emission Spectroscopy |  | ✓ |
| Bolometer – midplane array |  |  |
| CHERS – poloidal |  | ✓ |
| CHERS – toroidal | ✓ |  |
| Divertor Bolometer (LADA) |  | ✓ |
| Divertor visible cameras | ✓ |  |
| Dust detector |  |  |
| Edge deposition monitors [2] |  |  |
| Edge neutral density diag. |  | ✓ |
| Edge MIGs [2] |  | ✓ |
| Penning Gauges [2] |  | ✓ |
| Edge rotation diagnostic |  | ✓ |
| Fast cameras – divertor [2] | ✓ |  |
| Fast ion D_alpha - poloidal |  |  |
| Fast ion D_alpha - toroidal |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes [2] |  | ✓ |
| FIReTIP |  |  |
| Gas puff imaging – divertor |  |  |
| Gas puff imaging – midplane | ✓ |  |
| H cameras - 1D [2] | ✓ |  |
| Infrared cameras [2] | ✓ |  |
| Langmuir probes – divertor |  | ✓ |
| Langmuir probes – RF |  |  |
| Langmuir probes – RF ant. |  | ✓ |
| Magnetics – Diamagnetism |  | ✓ |
| Magnetics – Halo currents |  |  |
| Magnetics – RWM sensors |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MAPP |  |  |
| Mirnov coils – high f. |  |  |
| Mirnov coils – toroidal array |  |  |
| MSE-CIF |  |  |
| MSE-LIF |  |  |
| Neutron detectors [2] | ✓ |  |
| Plasma TV |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  | ✓ |
| Reflectometer – SOL |  | ✓ |
| SSNPA [2] |  |  |
| RF edge probes |  |  |
| Spectrometer – divertor |  | ✓ |
| Spectrometer – MonaLisa |  | ✓ |
| Spectrometer – VIPS |  | ✓ |
| Spectrometer – LOWEUS |  | ✓ |
| Spectrometer – XEUS |  | ✓ |
| TAE Antenna |  |  |
| Thomson scattering | ✓ |  |
| USXR – pol. Arrays |  | ✓ |
| USXR – multi-energy |  |  |
| USXR – TG spectr. |  |  |
| Visible Brems. det. [2] |  | ✓ |

Notes:

[1] Check marks in this table do not guarantee diagnostic availability. Check with diagnostic physicists or research operations management to ensure diagnostic coverage.

[2] In some cases, a given line represents multiple diagnostics. For instance, there are multiple SSNPAs, multiple IR cameras, multiple neutron detectors, and multiple Langmuir probe arrays.

Appendix #1: Allowed Neutral Beam Power vs. Pulse Duration

Heating of the primary energy ion dump limits the beam duration to that given in the following table:

Table A1: Beam power and pulse length as a function of acceleration voltage

Appendix #2: Table for neutron rate estimations:

Table A2: Neutron Emission Rate Calculator. Double click to open in excel for automatic calculation. Change only the blue cells.

TK Gray, et al.  Nuclear Fusion 54, 023001 (2014)

DA Russell, et al., Physics of Plasmas 22, 092311 (2015)

Assuming 10 shots at 0.9 MA that last ~ 2 seconds and 10 shots at 1.5 MA that last ~ 2 seconds.